## Slide 1: DION

Good afternoon. My name is Dion, this is Sherman and Zongzhe. Today we will be presenting Gappler.

Slide 2: DION

By 2030, one in four Singaporeans will be aged 65 or older. As we age, even everyday tasks can become more difficult. Something as simple as getting up, walking over to an object, and carrying it back can become a real challenge, especially when there’s a risk of falling. In fact, one in five seniors experiences a fall each year.

Imagine if you could simply look at what you want, ask for it, and have a robot bring it to you.

So, how do we make that happen? Here’s the big picture.

## Slide 2: DION

Imagine the robot starts in another room. You're in your room, and you can see the box you want. The robot can't, and it's never been in your room before.

You simply look at the box and say, "Get me the box."

The robot makes its way over, finds you,

and starts searching nearby, using your location as a clue.

But there might be several similar-looking boxes. So it checks whether the one it finds matches the box you were looking at. If it doesn't, it keeps searching.

Once it finds the right box, it picks it up.

Then it brings the box back to you.

This sounds simple but technically it poses big questions. Like how do we relay the user’s intent and find the desired object efficiently?

## Slide 3: DION

Well, we have a solution. Our solution hinges on 2 key papers that each solve part of the problem.

The first is FAM-HRI. It uses gaze and speech from the same pair of glasses to tell a fixed robotic arm what to pick. But it makes two key assumptions. The object is already visible and within reach, and the camera system that the robots uses is a fixed setup.

The second is HiCo-Nav. It focuses on helping robots explore their surroundings by building a memory graph of objects, including their 3D positions and visual features. But it doesn’t cover grasping, and it relies on speech alone. So if you say, “Find a box,” it might find any box, not necessarily the one you were looking at.

Our contribution is to bring these two ideas together. The robot explores its environment, finds the object you intended, picks it up, and brings it back to you.

We’ve broken this down into several milestones, which we’ll cover later. But first, let’s look at how the complete system is designed to work.

## Slide 4: ZONGZHE

The finished system works in five phases.

First, intent capture. The user speaks and looks. Speech gives us the word, like "box". Gaze tells us which box, and we cut a crop of it from the glasses image.

Second, explore and map. The robot drives through a room it has never seen, builds a 3D map, and remembers every object it passes. It also looks for the user, because the object is somewhere the user can see.

Third, search. The robot compares the objects it remembers with the user's target, and ranks them by how well they match, each with a position to drive to.

Fourth, approach and confirm. It drives close to the best match, and the camera on the arm checks that this is the object the user looked at. If not, it tries the next one.

Otherwise, the robot moves on to the final step, grasp. Once the object is confirmed, the arm picks it up.

## Slide 5: SHERMAN (lap 5

We didn’t build these phases from nothing. Each card shows the open-source tools the two papers used, and under each one is what we had to add to make it work for us. We will be going more into depth on each of these later on

The band below is what no paper gives us. The gaze match that joins the two papers. Clear ownership, so a change in one part never quietly breaks another. And hardware tested one step at a time, with two people at the robot.

Now we move on to each of our phases, starting with Phase one

## Slide 6: DION

The first phase is intent capture. We combine three sources of information from the glasses: speech, gaze, and video, to figure out which object the user wants.

Let’s walk through what happens when you say, “Find the box.”

First, a speech detector waits for you to finish speaking. Whisper converts your speech into text, and a small language model extracts the object name.

At the same time, gaze tracking follows your pupils to estimate where you’re looking.

We then give that object name to SAM 3, our image segmentation model. Since our prompt here is “box”, it detects all the boxes in the frame.

But as you can see, there are several boxes on the table. So how do we know which one you mean?

We use a voting system. Every millisecond, we find the mask closest to your gaze point. Over a one-second window, the same mask needs to win at least half the votes for us to select it.

We then crop the selected object with a small margin and send it to the robot.

So, how well does this work? In a recording with boxes between 1.4 and 2.4 metres away, we selected the correct box in 60 out of 61 frames.

In a full replay, with the user standing at four different locations 3 out of 4 commands produced a correct crop. The final miss is due to gaze accuracy. We found that it’s difficult to keep looking directly at a box for an extended period, and our gaze estimates still need further calibration.

You can see an example here. The gaze point landed on a stack of mats, which SAM 3 also classified as a box.

Keep this failure in mind. We’ll come back to it later.

Next, Sherman will explain how we establish the robot’s position.

## Slide 7: SHERMAN (lap 7)

Now, we move on to the second phase, where robot explores and maps out the room. We use FAST-LIVO2, the same method the HiCo-Nav paper uses. It combines the LiDAR, the motion sensor and the colour base camera to work out where the robot is in 3D. Later, the memory graph uses that to place each object in the room and decide which areas to explore.

This is the map it built of our lab. We pushed the base around this 26-metre loop by hand. The orange line is the path, and the ring is where it started and finished. After 26 m, the map's estimate came back within 12 mm of where it started. FAST-LIVO2 has no loop correction, so that is raw drift. We then ran the route twice more and compared both runs against this map. It was never off by more than about 7 centimetres, and whenever the robot stopped, it settled back to within a centimetre.

And it keeps up easily. A new LiDAR scan arrives every 100 milliseconds, and FAST-LIVO2 handles each one in about 12, on the CPU. That leaves the GPU free for everything else.x

FAST-LIVO2 was written for ROS 1, so we used a community port for ROS 2. That port had bugs that failed silently. They were fixed by checking it step by step against test runs.

The main limitation today is that the robot is pushed by hand. Next, Nav2, the navigation planner, drives the base on this map, first to a set point and then exploring. After the break, HiCo-Nav’s memory graph tells it where to explore first.

So the map tells us where the robot is. But the robot carries several cameras as Dion previously mentioned, and to use what they see, we need to know exactly where each one sits

## Slide 8: SHERMAN

To use what each sensor sees, we need four links between them. They’re numbered on the drawing, with a matching card for each on the right. Green means done, orange means part way, and grey means planned.

Link one is the arm on the base. It’s measured, so the arm and the base agree on where things are.

Link two is the wrist camera on the arm. It’s calibrated, so when the camera spots a grasp, the arm can reach it.

Link three is the glasses to the robot. That one’s still planned. It’s what will let the robot use your gaze to choose between two identical objects.

That leaves link four, the base camera to the LiDAR, and it matters most for what comes next. So far, it has been measured but not calibrated. We haven’t checked it against the sensors themselves. This is especially important because being off by just one degree shifts a point by about seven pixels, which is enough to give an object the wrong depth.

So next, I’ll calibrate it against a target. Then I’ll check it by drawing the LiDAR points over the camera image. If the edges line up, the calibration is right.

And that depth is exactly what the memory graph relies on.

## Slide 9: ZONGZHE

## Thanks, Sherman. With those poses and photos, I build the robot's memory of the room.

## (Top strip.) I’ll explain how the graph is built, from a frame in a recording into memory in five steps. First, keep a photo only after the robot moves a metre or turns 45 degrees. That keeps 29 of about 3500 frames. Second, find the objects in it with SAM 3. SAM 3 processes classes sequentially, so batching queries helped speed up object detection by about 5 times. Then, place each object in 3D with LiDAR depth, generate its vector embedding with CLIP, and merge repeat sightings mainly by where they are in 3D, with appearance as a smaller term.

## (Right.) Here is one kept photo on the right: SAM 3's masks on top, the same objects in 3D below.

## Slide 9.5: ZONGZHE

The graph does two jobs: maintaining the graph and answering prompts. The inputs and outputs of the graph are as shown. The graph stores anchors, the kept photos with the camera's position, and objects. Each object is linked to the anchors that saw it. Those links are the graph.

For now, it is built offline, from a recording.

I talked about how the graph detects objects quickly. However, making sure that the detected objects are accurate is just as, if not more important.

## Slide 10: ZONGZHE

## We made three changes to HiCo-Nav, so that one real chair becomes one object, in the right place.

## First, a noise filter. Using an example we found while testing; LiDAR is sparse, so it sees the floor through the gaps in a chair. Those readings pulled the chair's centre down to the floor. Now, a reading with a much nearer one beside it is dropped, and the chair is back at seat height.

## Second, one object per pixel. When initially detecting objects in images, same pixels sat in two objects. The graph merges by overlap, so shared pixels cause wrong merges. Now, each pixel stays in one mask only, preventing wrong merges.

## Third, we tuned the match radius, a parameter to decide if an object in 2 separate objects is identical, to balance between minimising duplicates and preventing wrong merges. Through experimental testing, we found that increasing the match radius from HiCo-Nav’s 2.5 centimetres to 10 centimetres cut duplicates from 8 to 3 with no wrong merges. At 15 centimetres, wrong merges started appearing.

## Now that the memory is right, the robot can ask it where to go.

## Slide 11: ZONGZHE

Search takes four steps. First, a small local language model turns the prompt into classes. "Find the chair" gives chair, plus table and desk, because chairs are usually near them. It got 20 of 22 test sentences right.

Second, a text encoder turns the prompt into a vector. Third, the graph keeps objects of those classes and ranks them by how close their CLIP image vectors are to it.

Lastly, it returns two ranked lists, targets first, then related, each hit with a 3D position. A wrong hit is rejected for the next.

This has not run on the whole drive yet. Today we ask with a sentence, so it finds any chair. To find the user's chair, we ask with the gaze crop.

Dion shows that comparison next.

## Slide 12: DION (lap9)

When the robot finds a potential object, it needs to check whether it’s the same one the user was looking at.

The challenge is that the glasses and the wrist camera see the object from very different angles. So how do we tell if they’re looking at the same thing?

We first tried keypoint matching, which looks for distinctive points in both images and tries to match them.

As you can see here, it failed to identify the correct box, and had its points all over the place. This was this same for 15 image pairs and we think that it was because the viewpoints are simply too different in terms of res, scale and noise.

So we tried a different approach. Instead of matching individual points, we compared the visual features of the entire object by comparing CLIP embeddings of both images.

This time, we identified the correct box in all 25 image pairs.

## Slide 12: DION

Following this, we integrated that matching directly into the grasp and the results are encouraging.

With two cardboard boxes, the keypoint system chose the wrong box in all 114 frames. Using the new CLIP embedding method, we identified the correct box in 266 out of 268 cases. With three boxes, we got all 334 right.

If nothing matches well enough, the arm waits rather than making a guess.

But theres a fundamental limitation with this method. When we tested two identical boxes, we got only six out of 12 right. This is expected as similar appearance would generate similar embeddings. That’s where the next glasses-to-robot calibration Sherman mentioned becomes important.

Now once we’ve selected the right target, we need the arm to grasp it. And on October 13th something miraculous happened.

## Slide 13: DION

This was the date we finally made our first successful grasp, recorded here.

Starting from a recorded spoken command, the arm succeeded in five out of eight attempts. Both failures at 54.5 centimetres were beyond its reach, so within reach, it succeeded in five out of six attempts.

The state machine validates each grasp request, then moves the wrist camera towards the target in 2 cm steps, checking the image along the way to correct for calibration and depth errors. Once close enough, the arm grasps and lifts the object, returns home, and holds it until the user says “Release.”

Safety was also a priority. We implemented a software stop that cancels motion in under a millisecond.

So far, we’ve used a simple approach. The arm moves to the centre of the object, then closes the gripper.

But how do we extend this to grasping at different angles?

## Slide 14: DION

AnyGrasp is a learned model that uses the object’s geometry to suggest where to place the gripper, at what angle, and with what confidence.

Here’s how we use it in our pipeline.

The arm stops 30 centimetres from the box, and the wrist camera captures a 3D snapshot.

SAM 3 identifies the box in the image, allowing us to isolate the points belonging to it.

AnyGrasp proposes several candidate grasps, which we rank by confidence. To avoid reacting to a noisy frame, we only commit to a candidate when it remains on top for five consecutive frames.

Next, the motion planner checks whether the arm can reach the grasp and finds a collision-free path. The arm moves to a point 30 centimetres in front of the grasp, then moves straight in.

In our real-arm tests in the ideal position, the grasps succeeded in three out of four attempts with the last closing just short of the box.

## Slide 15: DION

However, this approach starts to fall short when we try different object orientations, the following are our hypotheses.

First, the wrist camera only sees the front and top of the box, as you can see in this point cloud. It can’t see what’s behind, so AnyGrasp has less information to work with, making its grasp suggestions less reliable.

Another problem is that AnyGrasp considers the object’s shape, so when we allow steeper approach angles, most of the grasps suggested are beyond our arm’s reach.

Third, latency. Sometimes, the box’s position arrives two to four seconds late. This causes the arm to stop during its approach.

Finally, placement. Some grasps land too close to the edge of the object.

So, what’s next? We’re tackling these issues in stages. First, we check reachability before selecting a grasp to speed up the process of finding a candidate. We’re also exploring whether moving the arm to different viewpoints can give us a more complete view of the box. Another approach is to combine multiple camera views into a single 3D picture as we move closer. Finally, we’ll test 12 different box positions and orientations to measure how reliable our grasping really is.

With that, I’ll hand over to Sherman to give us an overview of our progress so far.

## Slide 15: SHERMAN (lap 14)

Now, a recap of everything we have talk about and how it fits in the pipeline.
 
At the start, we've built the step that turns a spoken word and a glance into a target crop, tested on recordings. This is then passed to wrist camera it to pick the right object That pick is wired into the grasp but so far only on replay. The arm itself already grasps on the real robot, and running the two together is next.

In the middle, we've built a 3D map of the lab. [Recorded drive, up to the memory graph] One drive through it was enough to build a memory of the objects in the room. [Grey link, Find the object] A search over that memory is built, and running it on this drive is next. [Grey link, Nav2] My pose already reaches Nav2, which is set up on the map and ready for its first drives.
 
What's left is four joins. First of all, The target crop has to feed the search.  Next, The memory graph has to tell the base where to look while it explores, so it checks the likely places first. Following that, A match has to become a drive goal. Finally, on arrival, the robot has to hand over to the wrist camera. With those four joins, the whole chain runs from a spoken word to a grasp on the desired object. 

Here's how that sits on our schedule.

## Slide 16: SHERMAN

This is our planned Gantt chart with all our major milestone listed at the side. We are currently on track to meet our goals and will continue work after the finals season.

## Slide 17: ZONGZHE

## Our tentative submitted budget is simple: one thousand five hundred dollars, all for compute, mostly for a GPU.

## Everything runs on one shared 16 gigabyte GPU. So, we had to work around some compute problems instead of solving. The paper's 8 billion parameter model does not fit next to SAM 3, so we use a small 1.5 billion model. SAM 3 is too slow for every frame, so we only segment a photo each metre the robot moves. And the models have never all run at once, so each part is tested alone.

## What we wanted was an RTX 3090 with 24 gigabytes. Used, it costs 1,100 to 1,500 dollars, before accounting for miscellaneous items, such as CPU and memory. New GPUs cost about 2,800. Both are over our 1,000-dollar GPU line.

## Slide 18: ZONGZHE

To close, we identified four areas of development moving forward.

First, the robot drives to the object the graph gives and parks where the arm can reach it.

Second, one orchestration layer: every phase reports what happened, and one node decides the next step.

Third, the reclarification node. When a request is unclear, say two identical boxes, it asks the user short questions until there is one object or a lead. At most three, and it never moves the robot.

Fourth, end-to-end testing, with success counted phase by phase.

Thank you. We are happy to take questions.